CS2100 Midterm CheatSheet

C Language (2s Complement is used!)

~ is NOT, ^ is XOR (Bitwise Operator)

! is boolean operator, not bitwise operator

%lf is used when you want to scan double

\t is used in printing to print a tab

Range of int: -231 to 231 – 1

Syntax of Structure

typedef struct {                           Use . if you want to access

members…..                         members of a structure.

} <structure name>;

We may do assignments with structure, meaning if we assign one structure to another structure variable, it will copy all the members of the structure into the respective members of the structure of the structure variable.

(*player_ptr).name /*legal*/

*player_ptr.name /*illegal as treated as *(player_ptr.name)/

player_ptr->name /*legal*/

Declaring an array of pointers: type *a[]

In ASCII table, it starts with capital letter first then lowercase letter. Each capital letter and it’s corresponding lowercase letter differs by a decimal value of 32

Number System

Conversion from decimal to base-R

Whole number: repeated division by R  (copy resultant digits down to up)

Fractional number: repeated multiplication by R (copy resultant digits up to down)

Negation of a number

1s: -x = 2n – x – 1 (n is number of bits of x in binary)

To get negation of x: invert all the bits of x (same for fraction)

2s: -x = 2n – x (n is number of bits of x in binary)

To get negation of x: invert all the bits of x and add 1

(if fraction, same thing, but plus 1 at the rightmost digit after the “.”)

Addition

2s:Perform addition as usual, ignore carry out of the MSB, check for overflow (occur when the resultant sign bit is different from that of the operands in the case where both operands have the same sign)

1s: Same as above, however if there is a carry out of the MSB, add 1 to the rightmost digit (same in fractional num)

Hexa: …, 9, A, B, C, D, E, F

Subtraction (a – b)

Convert b to its complement and perform addition.

Excess

*Binary rep. of x in excess-N = Binary rep. of (x + N)

*Dec rep. of x represented in excess-N: Convert x back to decimal and minus the result by N

IEE 754 Single-Precision Floating-Point Rep

1-bit sign, 8-bit exponent represented in EXCESS 127, 23-bit mantissa 
Decimal to IEE: Convert the number to binary, normalize it, and fill in the parts accordingly

Sign Extension

Unsigned: Copy all the bits and put it at the rightmost part, fill the unfilled front part with zeros

Signed: Copy the magnitude part and put it to the rightmost part, copy sign bit and put it at the first bit, then fill the in -between gap with zeros

1s and 2s: Copy all the bits and put it at the rightmost part, fill the unfilled front part with the sign bit of the original num.

MIPS (Immediate is a 2s complement)

Given k-bit addresses, you can have 2k addresses.

Branching

The immediate field is equal to the number of instructions you skip over, starting from the end of the branch instruction.  (if jump backwards then immediate is -ve, must count the branch instruction as one of the instruction to skip too!)

If branch is not taken: PCnew = PC + 4

If branch is taken: PCnew = (PC + 4) + (immediate x 4)

Jump

Given address of instruction, drop the first 4 bits and the last 2 bits, the rest will be your immediate field.

Getting NOT from NOR: nor x, x, $zero (x is the same register)

Getting NOT from XOR: xor x, x, y (y is the register that contain all 1s)

ANDI, ORI, XORI will zero extend the 16-bit immediate to the left!

LB and LH will sign extend the 8 bits and 16 bits that they fetched respectively from the memory and store it in the register

SB and SH will only store the 8 and 16 least significant bits in the register mentioned respectively into the memory

LUI shifts the given 16 bit immediate to the left by 16 bits and concatenate it with 16 zeros.

To generate a mask (i.e. keep the bits you want) using ANDI, you use bit 1 to keep the bit that you want and 0 for the bit you don’t want.

OR can be used to force certain bits to become 1.

ISA (MSB = most significant Byte)

Big-endian: MSB are stored in lower addresses

Little-endian: LSB are stored in lower addresses

Data Path

Instruction Execution Cycle: Fetch, Decode, Operand Fetch, Execute, Result Write (Store)

Multiplexer: Inputs: n lines, Control: m bits where n = 2m

Control (Note the following control signals aren’t complete)

RegDst: 0 -> choose rt; 1-> choose rd

ALUSrc: 0 -> choose RD2; 1-> choose SignExt(Inst[15:0])

MemToReg: 0 -> choose ALU result; 1-> choose Memory Read Data

PCSrc: 0 -> PC + 4; 1 -> (PC+4) + SignExt(Inst[15:0]) << 2

ALU Control

0000 AND | 0001 OR | 0010 add | 0110 subtract | 0111 slt

1100 NOR

ALUOp

00 lw/sw | 01 beq | 10 R-Type

Power of 2

27 = 128, 28 = 256, 29 = 512, 231 = 2147483648

Boolean Algebra

A + A’ = 1      A.A’ = 0 (Complement / Inverse)

X + X = X       X.X = X   (Idempotency)

(X’)’ = X   (Involution law)

X + X.Y = X     X.(X+Y) = X  (Absorption)

X + X’.Y = X + Y    X.(X’+Y) = X.Y (Absorption(variant))

X.Y + X’.Z + Y.Z  = X.Y + X’.Z (Consensus)

(X+Y).(X’+Z).(Y+Z) = (X+Y).(X’+Z) (Consensus)

A.B’ + A’.B = A⊕B

A.B + A’.B’ = (A⊕B)’ or A⊖B

Kmaps

Delay in Sum and Carry of a n-bit parallel adder

S = 2nt

C = (2n + 1)t // Maximum delay of the parallel adder

Decoder

The order of the outputs corresponds to the order of the minterms

Sequential Circuit (Characteristic Table)

S-R flip flop		              D-flip flop

Q(t + 1) = S + R’.Q                              Q(t + 1) = D

J-K flip flop                                   T flip-flop

Q(t + 1) = J.Q’ + K’.Q	               Q(t) = T.Q’ + T’.Q

Excitation Table

__________________________________________________

Pipelining (Instructions Execution Time)

Single-cycle: I x Tsingle cycle       Tsingle cycle = 1NTstage

Multiple-cycle: I x Average CPI x CTmulti

CTmulti  = max(Tstage), CPI = num of cycles per instruction

Pipeline: Numcycles x CTpipeline = (I + N – 1) x (max(Tstage) + Td)

N = num of stages in pipeline (MIPS is 5)

Td = pipeline overhead

Speeduppipeline= TimeseqTimepipeline

__________________________________________________

Early Branching: If no early branching (i.e. decide in MEM), then need to delay 3 cycles (refer top right corner!)

If early branching (i.e. decide in ID), delay 1 cycle)(But this depends on the register involved in the comparison!)

Branch Prediction:

Just continue as per normal

Without Forwarding Branch Taken at Stage 4

With Forwarding Brach Taken at Stage 2

Cache

Average Memory Access Time

(Hit rate x Hit Time) + [(1 – Hit rate) * Miss Penalty]

1 KB = 210 byes =1024 bytes, 1 MB = 220 bytes, 1 GB = 230 byes

Offset = 2X bytes. 1 word, 4 bytes, 4 word block = 16 bytes therefore offset = 4

Direct Map Given block size = 2M, num of blocks in cache = 2N

Block Index = Block number % num of blocks in cache

Mapping : Offset = M bits, Block Index = N bits

Tag = 32 – N – M bits

N-way Set associative

Set Index = Block number % number of sets in cache

Mapping: Offset = M bits, Set index = N bits

Tag = 32 – N – M bits     {num sets in cache = 2N}

Fully Associative: Offset = M bits, Tag = 32 – M bits

Writing Policies

Write Through: Write into cache & memory (Write into cache & write buffer, memory controller will write into memory from write buffer.

Write Back: Write into the cache only and only write into the memory if the block is evicted (and updated, by using dirty bit to indicate: 0 if not updated, 1 if updated)

Write MISS Policies

Write Allocate: Load the entire block into the cache, write only the word that is changed. Writing back to the memory depends on writing policies.

Write Around: Only write directly into the memory. Do not load the entire block into the cache.

Block Replacement Policies

Least Recently Used = remove oldest (For temporal locality)

Also LIFO, RR, Least Frequently Used

Cache Thrashing=keep replacing each other